Stage 16N Geometry and Cycle-Jump Workflow

Plate-with-hole Abaqus benchmark using a NEML-equivalent Chaboche UMAT

# Purpose

This document summarizes the Stage 16N benchmark geometry and the planned workflow for fixed and adaptive cycle-jump validation. The material model is kept fixed; the thesis contribution is the cycle-jump methodology and its accuracy-versus-cost evaluation on an inhomogeneous Abaqus model.

# Geometry Used in Stage 16N

| Item | Stage 16N setting |
| --- | --- |
| Benchmark type | Thin 3D plate with a central circular hole. |
| Dimensions | Length = 20.0, height = 10.0, thickness = 1.0. |
| Hole | Central hole with radius = 1.0 at the plate center. |
| Mesh spacing | Structured grid with dx = 0.25 and dy = 0.25. |
| Element type | C3D8 for Stage 16N NEML-equivalent Abaqus deck. |
| Nodes | 6642 generated nodes. |
| Elements | 3148 solid elements outside the hole. |
| Hole-ring set | 60 elements selected near the hole for local stress and STATEV tracking. |
| Boundary sets | 82 left-edge nodes and 82 right-edge nodes. |
| Anchors | Anchor A = node 1; Anchor B = node 3241, used to remove rigid-body motion. |

Figure 1. Stage 16 plate-with-hole geometry schematic.

Figure 2. Generated plate-with-hole mesh used for the benchmark.

# Loading, Material, and Output

| Item | Stage 16N setting |
| --- | --- |
| Material model | NEML-equivalent Chaboche cyclic plasticity UMAT. |
| State variables | 27 solution-dependent state variables in the Stage 16N UMAT. |
| Elastic constants | E = 200000.0, nu = 0.3. |
| Chaboche-style parameters | yield = 100.0, Q = 50.0, b = 5.0, three backstress pairs: C1/g1, C2/g2, C3/g3. |
| Cycle definition | Separate static step for each cycle, with AMP_ONE_CYCLE tabular loading. |
| Right edge loading | Cyclic displacement U1 amplitude = +/-0.10 on RIGHT_EDGE. |
| Left edge constraint | LEFT_EDGE fixed in U1; anchor nodes constrain remaining rigid-body modes. |
| Reference run | 1000 full non-jump cycles completed as the accepted baseline. |
| Selected field cycles | 1, 2, 10, 50, 100, 250, 500, 750, and 1000. |
| Tracked outputs | RF1-U1 hysteresis, loop area, local hole-ring stresses, and selected STATEV values. |

# Reference Status

The corrected full reference completed 1000 non-jump cycles.

Abaqus parallelism was verified as 1 MPI rank x 30 threads for the completed full reference.

Future production runs are configured to use 16 CPUs by default to reduce resource waste.

The reference CSV files provide the baseline for fixed and adaptive cycle-jump validation.

# Plan Flow Chart

Figure 3. Proposed Stage 16N workflow from benchmark setup to final cycle-jump validation.

# Key Decision Points

| Item | Stage 16N setting |
| --- | --- |
| Benchmark suitability | Accepted: global loop evolution and local hole STATEV evolution are significant. |
| Early-cycle treatment | Do not jump the first cycles aggressively; simulate the transient normally. |
| Delta N control | Use local STATEV and local stress variables in addition to global RF1 and loop area. |
| Validation order | Run fixed controlled jumps before the full adaptive cycle-jump workflow. |
| CPU policy | Use 16 CPUs by default and verify the Abaqus message file reports x 16 THREADS. |

# Immediate Next Tasks

Finish/check the 16-CPU benchmark and record the measured efficiency.

Analyze the 1000-cycle full reference data.

Compute the adaptive Delta N table from the full reference.

Run fixed jump validation cases inside the 1000-cycle window.

Implement adaptive cycle jumping after the fixed jumps pass the accuracy checks.